Please check which driver you should use:

1. Insert ASUS 167g into USB port.

2. cat /proc/usb/devices.

3. Check vendor id and product id of ASUS WLAN device.

4. Follow the README file in related directory to compile and install driver.

Vendor id	predicted	driver directory

0B05		1706		rt25USB

0B05		1707		rt25USB

0B05		1723		rt73

0B05		1724		rt73

For rt73, please add ASUS IDs (red words) in the rtmp_def.h.

#define RT73_USB_DEVICES { \

{USB_DEVICE(0x148f,0x2573)}, /* Ralink */      \

{USB_DEVICE(0x148f,0x2671)}, /* Ralink */      \

{USB_DEVICE(0x18e8,0x6196)}, /* Qcom */        \

{USB_DEVICE(0x18e8,0x6229)}, /* Qcom */        \

{USB_DEVICE(0x1044,0x8008)}, /* Gigabyte */    \

{USB_DEVICE(0x14b2,0x3c22)}, /* Conceptronic */    \

{USB_DEVICE(0x0db0,0x6877)}, /* MSI */         \

{USB_DEVICE(0x0db0,0xa874)}, /* MSI */         \

{USB_DEVICE(0x0db0,0xa861)}, /* MSI */         \

{USB_DEVICE(0x07b8,0xb21d)}, /* AboCom */      \

{USB_DEVICE(0x0769,0x31f3)}, /* Surecom */     \

{USB_DEVICE(0x1472,0x0009)}, /* Huawei-3Com */     \

{USB_DEVICE(0x1371,0x9022)}, /* CNet */        \

{USB_DEVICE(0x1371,0x9032)}, /* CNet */        \

{USB_DEVICE(0x1631,0xc019)}, /* Billionton */  \

{USB_DEVICE(0x1044,0x800a)}, /* Gigabyte */  \

{USB_DEVICE(0x1690,0x0722)}, /* Askey */                 \

{USB_DEVICE(0x0B05,0x1723)}, /* ASUS */                 \

{USB_DEVICE(0x0B05,0x1724)}, /* ASUS */                 \

{USB_DEVICE(0,0)}} /* end marker */