How to Create USB Flash Shell Disk

Step 1. Take one machines with Windows OS installed, plug in an USB flash disk.

Step 2. Launch “Windows Explorer” (or presses “Windows+ E”), and the USB flash disk plugged shall be shown in the arisen window as “Removable Disk” or so. Right click your mouse on it and select “Format…” to format this USB flash disk.

Step 3. In the dialog shown as below, make sure the following options are set properly.

(1) File System Choose “FAT32”

(2) Format Options Select “Quick Format”

And then press “Start” button to execute the formatting operation.

Click “OK” button for confirmation when the following dialog is shown, or click “Cancel” if you want to stop the operation.

When the formatting operation is finished, the dialog as below will be shown. Click “OK” to close the dialog.

Step 4. Unzip the compressed file of Shell boot package “EFI.Zip”. In the USB flash disk, create a path of “EFI\BOOT\”, and save the files “BOOTX64.EFI” in this path.

(“BOOTX64.EFI” must be saved in “ROOT:\EFI\BOOT\”!)

Step 5. Then this USB flash disk can be used as an USB Flash Shell Disk.

Shell Flash Instructions

Step 1. In Windows 8.1 x64 OS, unzip the compressed file of BIOS ROM package “Patch_Acer_BIOS_2.11_Shell.Zip” , and save the files “flashS.nsh”, “flash64.efi”, “isflash.bin” ,“FlashTXE.nsh”, “FwUpdLcl64.efi”, & “1089.bin” to the USB Flash Shell Disk. (Refer to “How to Create USB Flash Shell Disk”.)

Step 3. Please power on the target system, and press “F2” key to enter BIOS Setup Menu. In the “Boot” menu, switch USB Flash Shell Disk (USB HDD) to the first boot device in the “Boot priority order” by pressing F5/F6.

Step 4. Switch to the “Exit” menu, and choose “YES” in the “Exit Saving Changes”. The system will save changes and auto reboot.

Step 5. Boot into Shell with the USB Flash Shell Disk on the target machine, and change to the path where “flashS.nsh”, “flash64.efi”, “isflash.bin” ,“FlashTXE.nsh”, “FwUpdLcl64.efi”, & “1089.bin” were saved to in step 1.

(In this example, the files are unzipped and saved to Removable HardDisk “fs1:\” location.)

Step 6. Execute the batch file “flashS.nsh”, which will auto reboot and flash BIOS/EC.

Step 7. System will auto reboot, and start doing secure flash.

Notice:

While flashing BIOS on Secure flash windows please DO NOT remove AC Power Source. If warning message is shown as below, please check AC is plugged in or not.

Step 8. If everything works well, a few seconds after the update progress reaches 100%, the system will reboot.

Step 9. After the system reboot, please press “F2” key to enter BIOS Setup Menu. In the “Boot” menu, choose “YES” in the “Load Setup Defaults”.

Step 10. In the “Boot” menu, switch USB Flash Shell Disk (USB HDD) to the first boot device in the “Boot priority order” by pressing F5/F6 in order to update TXE in step11

Step 11. To update TXE, please boot into Shell with the USB Flash Shell Disk on the target machine, and change to the path where “flashS.nsh”, “flash64.efi”, “isflash.bin” ,“FlashTXE.nsh”, “FwUpdLcl64.efi”, & “1089.bin” were saved to in step1.

(In this example, the files are unzipped and saved to Removable HardDisk “fs1:\” location.)

Step 12. Execute the batch file “FlashTXE.nsh”, which will auto reboot and flash TXE.

Notice:

While flashing TXE, please DO NOT exit the process or power off the machine before the firmware update process ends.

Step 14. If everything works well, a few seconds after the update progress completed, the system will reboot.